# KNOWLEDGE INSTITUTE OF TECHNOLOGY

# (Autonomous)

SALEM–637504

RECORD NOTE BOOK

Register Number:

Certified that this is the bonafide record of work done by Selvan / Selvi 	          of the IV Semester

Branch during the Year 	in BE23AD551 – IT INFRASTRUCTURE

LABORATORY.

## Staff In-charge	Head of the Department

Submitted for the University Practical Examination held on …………………………..

Internal Examiner	External Examiner

CONTENT

| Ex. No. | Date | Name of the Exercise | Marks | Signature |
| --- | --- | --- | --- | --- |
| 1 |  | Assemble a PC and Verify Hardware via BIOS/UEFI |  |  |
| 2 |  | Upgrade RAM and Clone HDD/SSD to NVMe |  |  |
| 3 |  | Install and configure printer and scanner and troubleshooting driver and connectivity issues |  |  |
| 4 |  | Create dual-boot Vm with windows and Linux using Partioning |  |  |
| 5 |  | Installation and Basic Commands of Linux Operating System |  |  |
| 6 |  | Install and update and roll back while software checking application compatibility |  |  |
| 7 |  | Configure windows update and WSUS for patch management |  |  |
| 8 |  | Deploy antivirus and endpoint security agents configure scanning policies |  |  |
| 9 |  | Enter assets ITSM tool and manage tickets with sla tracking |  |  |
| 10 |  | Implement backup and restore policies for endpoints to ensure data integrity |  |  |

| EX:NO: 1 | Assemble a PC and Verify Hardware via BIOS/UEFI |
| --- | --- |

AIM:

To assemble a desktop computer system from individual hardware components and verify proper installation and functioning using BIOS/UEFI settings.

## Apparatus / Requirements

- Motherboard

- CPU (Processor)

- RAM (Memory modules)

- SMPS (Power Supply Unit)

- Storage Device (HDD/SSD)

- Cabinet (Case)

- Monitor, Keyboard, Mouse

- Thermal paste

- Screwdriver set

## PROCEDURE

### Step 1: Preparation

- Ensure all components are available and working.

- Work on a non-static surface (use anti-static precautions).

### Step 2: Install CPU

- Place the motherboard on a flat surface.

- Open CPU socket latch.

- Align CPU with socket (check notch/triangle mark).

- Gently place CPU and lock the latch.

- Apply thermal paste and fix CPU cooler.

### Step 3: Install RAM

- Open RAM slot clips.

- Align RAM notch with slot.

- Insert RAM firmly until clips lock automatically.

### Step 4: Mount Motherboard

- Place motherboard inside cabinet.

- Align with screw holes.

- Secure using screws.

### Step 5: Install Power Supply (SMPS)

- Fix SMPS in cabinet.

- Connect main power cable (24-pin) to motherboard.

- Connect CPU power cable (4/8-pin).

### Step 6: Install Storage Device

- Mount HDD/SSD in drive bay.

- Connect SATA cable to motherboard.

- Connect power cable from SMPS.

### Step 7: Connect Front Panel & Peripherals

- Connect power switch, reset switch, LEDs.

- Attach keyboard, mouse, and monitor.

### Step 8: Power On System

- Switch on power supply.

- Press power button.

- Observe system startup.

### Step 9: Enter BIOS/UEFI

- Press Del / F2 / F10 during boot.

- Enter BIOS/UEFI setup.

### Step 10: Verify Hardware

- Check CPU details.

- Verify installed RAM capacity.

- Confirm storage detection.

- Check system temperature and fan status.

Diagram

Result

The PC was successfully assembled, and all hardware components were verified using BIOS/UEFI. The system is functioning properly.

| EX: NO: 2 | Upgrade RAM and Clone HDD/SSD to NVMe |
| --- | --- |

AIM:

To upgrade system RAM and clone an existing HDD/SSD to an NVMe SSD for improved system performance.

## Apparatus / Requirements

- Desktop/Laptop system

- RAM module (compatible)

- NVMe SSD

- Screwdriver set

- Cloning software (e.g., Macrium Reflect / Clonezilla)

- SATA/NVMe connectors

## PROCEDURE

### RAM Upgrade

- Shut down the system and disconnect power.

- Open the cabinet/laptop back panel.

- Locate RAM slots on the motherboard.

- Open the slot clips.

- Align RAM notch with slot.

- Insert RAM firmly until clips lock.

- Close the system and power it on.

- Verify RAM in system properties or BIOS.

### NVMe Installation

- Locate M.2 slot on motherboard.

- Insert NVMe SSD at a 30° angle.

- Gently press and secure with screw.

- Boot the system.

### Part C: Disk Cloning

- Install cloning software (e.g., Macrium Reflect / Clonezilla).

- Launch the software.

- Select source disk (HDD/SSD).

- Select destination disk (NVMe SSD).

- Start cloning process.

- Wait until cloning completes.

- Restart system.

### Part D: Set Boot Priority

- Enter BIOS/UEFI (Del/F2 key).

- Go to Boot settings.

- Set NVMe SSD as primary boot device.

- Save and exit.

DIAGRAM

RESULT

The RAM was successfully upgraded and the existing system was cloned to NVMe SSD. The system performance improved significantly.

| EX: NO: 3 | INSTALL AND CONFIGURE PRINTER AND SCANNER AND TROUBLESHOOTING DRIVER AND CONNECTIVITY ISSUES |
| --- | --- |

# AIM:

To install, configure, and test a printer/scanner, and troubleshoot common driver and connectivity issues.

Apparatus / Requirements

- Desktop/Laptop (Windows/Linux OS)

- Printer or All-in-One Printer/Scanner

- USB cable / Wi-Fi network connection

- Device drivers (CD/downloaded from manufacturer website)

- Internet connection

- Paper for test print

PROCEDURE

Printer Installation (USB Method)

- Connect the printer to the computer using a USB cable.

- Power ON the printer.

- Open Control Panel → Devices and Printers.

- Click Add a Printer.

- Select the detected printer from the list.

- If not detected, choose Add manually and select the correct port.

- Install drivers (automatic or from CD/download).

- Complete the setup.

Printer Installation (Wi-Fi Method)

- Connect the printer to the Wi-Fi network using its control panel.

- Ensure the computer is on the same network.

- Go to Settings → Printers & Scanners → Add Device.

- Select the wireless printer.

- Install required drivers.

- Finish setup.

Scanner Setup

- Install scanner drivers/software.

- Open scanning software (e.g., Windows Scan).

- Place document on scanner glass.

- Select scan type (color/grayscale).

- Click Scan and save the file.

Testing

- Right-click the printer → Printer Properties.

- Click Print Test Page.

- Verify output quality.

Troubleshooting

- Check power and cable connections

- Restart printer and system

- Clear print queue

- Update/reinstall drivers

- Check network connectivity (for Wi-Fi printers)

- Set printer as default

### Result

The printer/scanner was successfully installed and configured. Test print and scanning operations were performed, and common issues were identified and resolved

| EX: NO: 4 | CREATE DUAL-BOOT VM WITH WINDOWS AND LINUX USING PARTIONING |
| --- | --- |

# AIM:

To install and configure a dual boot system with Windows and Linux using disk partitioning.

Apparatus / Requirements

- Computer/Laptop (or Virtual Machine software like VirtualBox / VMware Workstation)

- Windows OS (pre-installed or ISO file)

- Linux distribution ISO (e.g., Ubuntu)

- Minimum 4 GB RAM (8 GB recommended)

- Minimum 40 GB free disk space

- Bootable USB (for physical system)

Procedure

Prepare Disk Partition

- Boot into Windows.

- Open Disk Management.

- Select main drive → Right-click → Shrink Volume.

- Create free unallocated space (at least 20 GB for Linux).

Create Virtual Machine (Optional Method)

- Install VirtualBox.

- Create a new VM:

  - Name: Linux

  - Type: Linux

  - RAM: 2–4 GB

- Create virtual hard disk (40 GB).

Install Linux

- Boot system (or VM) using Linux ISO (e.g., Ubuntu).

- Select Install Ubuntu.

- Choose “Install alongside Windows” OR “Something else”.

- Create partitions:

  - Root / (20 GB)

  - Swap (2–4 GB)

- Proceed with installation.

Install Bootloader

- GRUB bootloader installs automatically.

- Restart system after installation.

Boot Selection

- On startup, GRUB menu appears.

- Select:

  - Ubuntu (Linux)

  - Windows

Diagram

Result

A dual boot system with Windows and Linux was successfully created using disk partitioning, and both operating systems were accessed through the GRUB bootloader

| EX: NO: 5 | PERFORM LINUX TERMINAL COMMANDS AND WRITE BASIC SHELL SCRIPTS FOR AUTOMATION |
| --- | --- |

Aim

To install a Linux operating system and perform basic file and directory management commands.

Apparatus / Requirements

- Computer/Laptop

- Linux ISO (e.g., Ubuntu)

- Virtualization software like VirtualBox (optional)

- Minimum 4 GB RAM

- Internet connection

Basic Linux Commands

- Check current directory

Pwd

- List files and directories

ls

- Change directory

cd folder_name

- Create directory

mkdir test

- Create file

touch file.txt

- View file content

cat file.txt

- Copy files

cp file.txt copy.txt

- Move/Rename file

mv file.txt newfile.txt

- Delete file

rm file.txt

- Clear screen

clear

Result

Linux OS was successfully installed, and basic commands were executed to manage files and directori

| EX: NO:6 | INSTALL AND UPDATE AND ROLL BACK WHILE SOFTWARE CHECKING APPLICATION COMPATIBILITY |
| --- | --- |

Aim

To install software, perform updates, roll back to a previous version, and verify application compatibility with the system.

Apparatus / Requirements

- Computer/Laptop (Windows/Linux)

- Software application (e.g., Google Chrome or VLC Media Player)

- Internet connection

- Administrator access

PROCEDURE

Software Installation

- Download software (e.g., VLC Media Player).

- Run the installer file.

- Follow installation steps (Next → Install → Finish).

- Launch the application to verify installation.

Software Update

- Open the installed application.

- Go to Help → Check for Updates (if available).

- Download and install updates.

- Restart application after update.

Compatibility Check

- Right-click application → Properties.

- Go to Compatibility tab.

- Run in compatibility mode (if needed).

- Check performance and functionality.

Rollback Process

- Open Control Panel → Programs and Features.

- Select the installed software.

- Click Uninstall/Change.

- Reinstall previous version (older setup file).

- Verify system stability.

.

Result

Software was successfully installed, updated, and tested for compatibility. Rollback was performed to restore a stable version when required.

| EX: NO: 7 | CONFIGURE WINDOWS UPDATE AND WSUS FOR PATCH MANAGEMENT |
| --- | --- |

Aim

To configure Windows Update settings and set up WSUS (Windows Server Update Services) for centralized patch management in a network.

Apparatus / Requirements

- Windows Server system (for WSUS)

- Client Windows PCs

- Internet connection

- Administrator privileges

- Windows Server

- Windows Update

- WSUS

Procedure

Configure Windows Update (Client System)

- Open Settings → Update & Security.

- Go to Windows Update.

- Click Check for Updates.

- Enable:

  - Automatic updates

  - Active hours configuration

- Restart system after updates are installed.

Install WSUS (Server Side)

- Open Server Manager.

- Click Add Roles and Features.

- Select WSUS role.

- Choose required components:

  - Windows Internal Database

  - Update Services

- Specify update storage location.

- Complete installation and restart server.

Configure WSUS Server

- Open WSUS Console.

- Configure synchronization settings:

  - Microsoft Update source

  - Language and products selection

- Start synchronization process.

- Approve required updates manually.

Connect Client Machines to WSUS

- Open Group Policy Editor (gpedit.msc).

- Navigate to:

Computer Configuration → Administrative Templates → Windows Components → Windows Update

- Enable: Specify intranet Microsoft update service location.

- Enter WSUS server URL:

http://<WSUS_Server_IP>:8530

- Apply policy and restart client system.

Testing

- Run command on client:

gpupdate /force

- Open Windows Update.

- Verify updates are coming from WSUS server.

Result

Windows Update was configured successfully on client systems and WSUS was installed and configured on the server for centralized

| EX: NO: 8 | DEPLOY ANTIVIRUS AND ENDPOINT SECURITY AGENTS CONFIGURE SCANNING POLICIES |
| --- | --- |

Aim

To install and deploy antivirus/endpoint security agents on systems and configure scanning policies for malware protection.

Apparatus / Requirements

- Windows/Linux computer systems

- Internet connection or offline installer

- Administrator privileges

- Central management console (if available)

- Antivirus software such as Windows Defender or enterprise endpoint tools like Microsoft Defender for Endpoint

Procedure

Install Antivirus / Endpoint Agent

- Download or access antivirus installer (e.g., Windows Defender or endpoint agent).

- Run installer as Administrator.

- Accept license agreement.

- Complete installation.

- Restart system if required.

Enable Real-Time Protection

- Open Windows Security / Antivirus Console.

- Go to Virus & Threat Protection.

- Turn ON:

  - Real-time protection

  - Cloud-delivered protection

  - Automatic sample submission

Configure Scanning Policies

- Open Security Settings / Policy Manager.

- Select Scan Options.

- Configure:

  - Quick Scan schedule (daily)

  - Full Scan schedule (weekly)

  - Custom scan paths (important folders)

- Enable automatic updates for virus definitions.

Deploy Endpoint Security Agent (Network Setup)

- Access central management console (e.g., Microsoft Defender for Endpoint).

- Create device group.

- Generate installation package/script.

- Deploy agent to client machines using:

  - Group Policy

  - Script deployment

  - Manual installation

- Verify agent connectivity in dashboard.

Testing

- Run Quick Scan and Full Scan.

- Check threat detection logs.

- Verify endpoint status in management console.

Result

Antivirus and endpoint security agents were successfully installed and configured. Scanning policies were applied, and system security was verified through scanning and monitoring

| EX: NO: 9 | ENTER ASESSETS ITSM TOOL AND MANAGE TICKETS WITH SLA TRACKING |
| --- | --- |

Aim

To explore an IT Service Management (ITSM) tool, manage IT assets, create and track service tickets, and monitor them using SLA (Service Level Agreement) tracking.

Apparatus / Requirements

- Computer/Laptop with internet access

- ITSM tool (e.g., ServiceNow or similar ITSM platform)

- Admin/User credentials

- Web browser

Procedure

Login to ITSM Tool

- Open browser and access ITSM portal (e.g., ServiceNow).

- Enter username and password.

- Navigate to the dashboard.

Asset Management

- Go to Asset Management module.

- Add new asset:

  - Asset name (Laptop/Printer)

  - Serial number

  - Category (Hardware/Software)

- Assign asset to user or department.

- Save and verify asset entry.

Ticket Creation

- Go to Incident Management / Service Desk.

- Click Create New Ticket.

- Fill details:

  - Issue description

  - Priority (Low/Medium/High/Critical)

  - Category (Hardware/Software/Network)

- Submit ticket.

Ticket Assignment and Tracking

- Assign ticket to support team or technician.

- Monitor ticket status:

  - Open

  - In Progress

  - On Hold

  - Resolved

SLA Configuration and Monitoring

- Define SLA rules:

  - Response time (e.g., 2 hours for high priority)

  - Resolution time (e.g., 24 hours)

- Apply SLA to ticket categories.

- Monitor SLA compliance dashboard.

- Check alerts for SLA breach warnings.

### Result

ITSM tool was successfully accessed. IT assets were managed, tickets were created and tracked, and SLA monitoring was implemented for service performance evaluation

| EX: NO: 10 | IMPLEMENT BACKUP AND RESTORE POLICIES FOR ENDPOINTS TO ENSURE DATA INTEGRITY |
| --- | --- |

Aim

To configure backup and restore policies on endpoint systems to ensure data integrity and recovery in case of data loss or system failure.

Apparatus / Requirements

- Windows/Linux endpoint system

- External storage device / cloud storage

- Administrator privileges

- Backup software or built-in tools

- Windows Backup and Restore or similar tool

- Optional cloud backup service

PROCEDURE

Configure Backup Policy

- Open backup tool (e.g., Windows Backup and Restore).

- Click Set up Backup.

- Select backup location:

  - External drive OR

  - Network location OR

  - Cloud storage

- Choose data to back up:

  - Files and folders

  - System image (optional)

- Set backup schedule:

  - Daily / Weekly / Monthly

- Save and start backup process.

Verify Backup

- Check backup completion status.

- Ensure backup files are stored in selected location.

- Confirm file integrity by opening sample files.

Restore Data

- Open backup utility.

- Select Restore Files option.

- Choose backup version/date.

- Select files/folders to restore.

- Restore to original or new location.

- Verify restored data integrity.

System Restore (Optional)

- Open System Restore option.

- Select restore point.

- Confirm restore operation.

- Restart system after completion.

Result

Backup and restore policies were successfully implemented on endpoint systems, ensuring data integrity and enabling recovery of data in case of failure or loss